Tel: +44 (01283 224 706

Web: www.krakenoffshore.co.uk

Email: info@krakenoffshore.co.uk

Kraken Offshore Ltd

Doc No:		N/A

Doc Title:	Convention on the International Regulations for Preventing Collisions at Sea, 1972 (COLREGS)

| 3 |  |  |
| --- | --- | --- |
| 2 |  |  |
| 1 |  |  |
| 0 | 13/11/23 | Issued for the benefit of seafarers |
| Rev No. | Date | Description |
| The accountable owner of this document is the: Managing Director |  |  |
| Comments: |  |  |

Prepared by: Lee Wilkes Managing Director	Date: 13/11/2023

Reviewed by: Eleanor Wilkes Director		Date: 13/11/2023

Approved by: Lee Wilkes Managing Director	Date: 13/11/2023

This document and its contents are the sole property of Kraken Offshore Ltd. This document is uncontrolled when printed, downloaded, or copied. Any distribution or use of this document shall be at the discretion of Kraken Offshore Ltd. To receive a copy of the current controlled document contact Kraken Offshore Ltd

Convention on the International Regulations for Preventing Collisions at Sea, 1972

(as amended by Resolutions A.464(XII), A.626(15), A.678(16), A.736(18), A.910(22)) and A.1085(28))

TABLE OF CONTENTS

PART A

GENERAL

1 - 3

PART B

STEERING AND SAILING RULES

Section I

Section II

Section III

Conduct of vessels in any conditions of visibility 
Conduct of vessels in sight of one another 
Conduct of vessels in restricted visibility

4 - 10

11 - 18

19

PART C

LIGHTS AND SHAPES

20 - 31

PART D

PART E

PART F

ANNEX I

SOUND AND LIGHT SIGNALS

EXEMPTIONS

VERIFICATION OF COMPLIANCE WITH THE PROVISIONS OF

THE CONVENTION

POSITIONING AND TECHNICAL DETAILS OF LIGHTS AND SHAPES

32 - 37

38

39 - 41

ANNEX II

ADDITIONAL  SIGNALS  FOR  FISHING   VESSELS  FISHING  IN  CLOSE PROXIMITY

ANNEX III

TECHNICAL DETAILS OF SOUND SIGNAL APPLIANCES

ANNEX IV

DISTRESS SIGNALS

PART A - GENERAL

Rule 1

Application

(a)

(b)

(c)

(d)

(e)

These  Rules  shall  apply  to  all  vessels  upon  the  high  seas  and  in  all  waters  connected 
therewith navigable by seagoing vessels.

Nothing  in  these  Rules  shall  interfere  with  the  operation  of  special  rules  made  by  an 
appropriate  authority for roadsteads, harbours, rivers, lakes or inland waterways connected 
with the high seas  and navigable by seagoing vessels. Such special rules shall conform as 
closely as possible to these Rules.

Nothing  in  these  Rules  shall  interfere  with  the  operation  of  any  special  rules  made  by 
the  Government of any State with respect to additional station or signal lights, shapes or 
whistle  signals for ships of war and vessels proceeding under convoy, or with respect to 
additional station  or signal lights or shapes for fishing vessels engaged in fishing as a fleet. 
These additional station or signal lights, shapes or whistle signals shall, so far as possible, be 
such that they cannot be mistaken for any light, shape or signal authorised elsewhere under 
these Rules.

Traffic separation schemes may be adopted by the Organization for the purpose of these 
Rules.

Whenever the Government concerned shall have determined that a vessel of any special 
construction or purpose cannot comply with the provisions of any of these Rules with respect 
to  the number, position, range or arc of visibility of lights or shapes, as well as to the 
disposition and characteristics of sound-signalling appliances, such vessel shall comply with 
such other   provisions in regard to the number, position, range or arc of visibility of lights or 
shapes, as well  as to the disposition and characteristics of sound-signalling appliances, as her 
Government shall have determined to be the closest possible compliance with these Rules in 
respect of that vessel.

Rule 2 
Responsibility

(a)

(b)

Nothing in these Rules shall exonerate any vessel, or the owner, master or crew thereof, from 
the  consequences  of any neglect  to comply with  these  Rules  or  of the neglect  of any 
precaution which  may be required by the ordinary practice of seamen, or by the special 
circumstances of the case.

In construing and complying with these Rules due regard shall be had to all dangers of 
navigation  and collision and to any special circumstances, including the limitations of the 
vessels involved, which may make a departure from these Rules necessary to avoid immediate 
danger.

Rule 3

General definitions

For the purpose of these Rules, except where the context otherwise requires:

(a)

(b)

(c)

(d)

(e)

(f)

(g)

The word “vessel”  includes  every  description  of  water  craft,  including non-displacement

craft,  WIG craft and seaplanes, used or capable of being used as a means of transportation 
	on water.

The term “power-driven vessel” means any vessel propelled by machinery.

The term “sailing vessel” means any vessel under sail provided that propelling machinery,

if  fitted, is not being used.

The term “vessel engaged in fishing” means any vessel fishing with nets, lines, trawls or

other  fishing  apparatus  which  restrict  manoeuvrability,  but  does  not  include  a  vessel 
	fishing with trolling lines or other fishing apparatus which do not restrict manoeuvrability.

The word “seaplane” includes any aircraft designed to manoeuvre on the water.

The term “vessel not under command” means a vessel which through some exceptional

circumstance is unable to manoeuvre as required by these Rules and is therefore unable to 
	keep out of the way of another vessel.

The term “vessel restricted in her ability to manoeuvre” means a vessel which from the nature

of  her work is restricted in her ability to manoeuvre as required by these Rules and is

therefore  unable to keep out of the way of another vessel. The term “vessels restricted in 
	their ability to manoeuvre” shall include but not be limited to:

(i)          a vessel engaged in laying, servicing or picking up a navigation mark, submarine

(ii)

(iii)

(iv)

(v)

(vi)

cable or pipeline;

a vessel engaged in dredging, surveying or underwater operations;

a   vessel   engaged   in   replenishment   or   transferring   persons,   provisions   or 
cargo  while underway;

a vessel engaged in the launching or recovery of aircraft;

a vessel engaged in mine clearance operations;

a vessel engaged in a towing operation such as severely restricts the towing vessel 
and her tow in their ability to deviate from their course.

(h)

(i)

(j)

(k)

(l)

(m)

The term “vessel constrained by her draught” means a power-driven vessel which, because of

her  draught in relation to the available depth and width of navigable water, is severely 
restricted in her ability to deviate from the course she is following.

The word “underway” means that a vessel is not at anchor, or made fast to the shore, or 
aground.

The words “length” and “breadth” of a vessel mean her length overall and greatest breadth.

Vessels shall be deemed to be in sight of one another only when one can be observed visually 
from the other.

The term “restricted visibility” means any condition in which visibility is restricted by fog, mist,

falling snow, heavy rainstorms, sandstorms or any other similar causes.

The  term  “Wing-in-Ground  (WIG)  craft”  means  a  multimodal  craft  which,  in  its  main

operational mode, flies in close proximity to the surface by utilizing surface-effect action.

PART B - STEERING AND SAILING RULES

Section I - Conduct of vessels in any condition of visibility

Rule 4

Application

Rules in this Section apply in any condition of visibility.

Rule 5

Look-out

Every vessel shall at all times maintain a proper look-out by sight and hearing as well as by all 
available  means appropriate in the prevailing circumstances and conditions so as to make a full 
appraisal of the situation and of the risk of collision.

Rule 6

Safe speed

Every vessel shall at all times proceed at a safe speed so that she can take proper and effective 
action  to avoid collision and be   stopped   within   a   distance   appropriate   to   the   prevailing 
circumstances and conditions.

In determining a safe speed the following factors shall be among those taken into account:

(a)

By all vessels:

(i) 
(ii) 
(iii)

(iv)

(v)

(vi)

the state of visibility;

the traffic density including concentrations of fishing vessels or any other vessels;

the manoeuvrability of the vessel with special reference to stopping distance and turning

ability in the prevailing conditions;

at night the presence of background light such as from shore lights or from back 
scatter of her own lights;

the state of wind, sea and current, and the proximity of navigational hazards;

the draught in relation to the available depth of water.

(b)

Additionally, by vessels with operational radar:

(i) 
(ii) 
(iii) 
(iv)

(v)

(vi)

the characteristics, efficiency and limitations of the radar equipment;

any constraints imposed by the radar range scale in use;

the effect on radar detection of the sea state, weather and other sources of interference; 
the possibility that small vessels, ice and other floating objects may not be detected

by radar at an adequate range;

the number, location and movement of vessels detected by radar;

the more exact assessment of the visibility that may be possible when radar is used to 
determine the range of vessels or other objects in the vicinity.

Rule 7

Risk of collision

(a)

(b)

(c)

(d)

Every vessel shall  use all available means appropriate  to the prevailing circumstances and 
conditions to determine if risk of collision exists. If there is any doubt such risk shall be 
deemed to exist.

Proper use  shall  be  made  of  radar  equipment  if  fitted  and  operational,  including  long- 
range scanning to obtain early warning of risk of collision and radar plotting or equivalent 
systematic observation of detected objects.

Assumptions  shall  not  be  made  on  the  basis  of  scanty  information,  especially  scanty 
radar information.

In determining if risk of collision exists the following considerations shall be among those 
taken into account:

(i)         such risk shall be deemed to exist if the compass bearing of an approaching vessel

does not appreciably change;

(ii)         such  risk  may  sometimes  exist  even  when  an  appreciable  bearing  change  is

evident,   particularly  when  approaching  a  very  large  vessel  or  a  tow  or  when 
approaching a vessel at close range.

Rule 8

Action to avoid collision

(a)

(b)

(c)

(d)

(e)

(f)

Any action taken to avoid collision shall be taken in accordance with the Rules of this Part 
and shall, if the circumstances of the case admit, be positive, made in ample time and with due 
regard to the observance of good seamanship.

Any alteration of course and/or speed to avoid collision shall, if the circumstances of the 
case admit, be large enough to be readily apparent to another vessel observing visually or by 
radar; a succession of small alterations of course and/or speed should be avoided.

If there is sufficient sea-room, alteration of course alone may be the most effective action to 
avoid  a close-quarters situation provided that it is made in good time, is substantial and does 
not result in another close-quarters situation.

Action taken to avoid collision with another vessel shall be such as to result in passing at a 
safe distance. The effectiveness of the action shall be carefully checked until the other vessel is 
finally past and clear.

If necessary to avoid collision or allow more time to assess the situation, a vessel shall slacken 
her speed or take all way off by stopping or reversing her means of propulsion.

(i)        A vessel which, by any of these Rules, is required not to impede the passage or

(ii)

(iii)

safe  passage of another vessel shall, when required by the circumstances of the case, 
take early action to allow sufficient sea-room for the safe passage of the other vessel.

A vessel required not to impede the passage or safe passage of another vessel is 
not  relieved of this obligation if approaching the other vessel so as to involve risk of 
collision  and shall, when taking action, have full regard to the action which may be 
required by the Rules of this Part.

A vessel the passage of which is not to be impeded remains fully obliged to comply 
with the Rules of this Part when the two vessels are approaching one another so as to 
involve risk of collision.

Rule 9

Narrow channels

(a)

(b)

(c)

(d)

A vessel proceeding along the course of a narrow channel or fairway shall keep as near to 
the  outer limit of the channel or fairway which lies on her starboard side as is safe and 
practicable.

A vessel of less than 20 metres in length or a sailing vessel shall not impede the passage of a 
vessel which can safely navigate only within a narrow channel or fairway.

A vessel engaged in fishing shall not impede the passage of any other vessel navigating within 
a narrow channel or fairway.

A vessel shall not cross a narrow channel or fairway if such crossing impedes the passage of 
a vessel which can safely navigate only within such channel or fairway. The latter vessel may 
use the sound signal prescribed in Rule 34(d) if in doubt as to the intention of the crossing 
vessel.

(e)

(i)        In a narrow channel or fairway when overtaking can take place only if the vessel to 
	be  overtaken  has  to take  action  to  permit  safe  passing, the  vessel  intending  to 
	overtake shall indicate her intention by sounding the appropriate signal prescribed in

Rule 34(c)(i). The vessel to be overtaken shall, if in agreement, sound the appropriate

signal prescribed in  Rule 34(c)(ii) and take steps to permit safe passing. If in doubt she

may sound the signals prescribed in Rule 34(d).

(f)

(g)

(ii)       This Rule does not relieve the overtaking vessel of her obligation under Rule 13.

A vessel nearing a bend or an area of a narrow channel or fairway where other vessels may 
be obscured by an intervening obstruction shall navigate with particular alertness and caution 
and shall sound the appropriate signal prescribed in Rule 34(e).

Any vessel shall, if the circumstances of the case admit, avoid anchoring in a narrow channel.

Rule 10

Traffic separation schemes

(a)

(b)

This Rule applies to traffic separation schemes adopted by the Organization and does not 
relieve  any vessel of her obligation under any other Rule.

A vessel using a traffic separation scheme shall:

(i)         proceed in the appropriate traffic lane in the general direction of traffic flow for that

lane;

(ii)         so far as practicable keep clear of a traffic separation line or separation zone;

(iii)        normally join or leave a traffic lane at the termination of the lane, but when joining 
	or leaving from either side shall do so at as small an angle to the general direction of

traffic flow as practicable.

(c)

(d)

A vessel shall, so far as practicable, avoid crossing traffic lanes but if obliged to do so shall 
cross on a heading as nearly as practicable at right angles to the general direction of traffic 
flow.

(i)        A  vessel  shall  not  use  an  inshore  traffic  zone  when  she  can  safely  use  the

appropriate  traffic lane within the adjacent traffic separation scheme. However, vessels 
of less than 20 metres in length, sailing vessels and vessels engaged in fishing may 
use the inshore traffic zone.

(ii)       Notwithstanding sub-paragraph (d) (i), a vessel may use an inshore traffic zone when 
	en route to or from a port, offshore installation or structure, pilot station or any other

place situated within the inshore traffic zone, or to avoid immediate danger.

(e)

A vessel other than a crossing vessel or a vessel joining or leaving a lane shall not normally 
enter a separation zone or cross a separation line except:

(i) 
(ii)

in cases of emergency to avoid immediate danger; 
to engage in fishing within a separation zone.

(f)

(g)

(h)

(i)

(j)

(k)

(l)

A vessel navigating in areas near the terminations of traffic separation schemes shall do so 
with particular caution.

A vessel shall so far as practicable avoid anchoring in a traffic separation scheme or in areas 
near its terminations.

A vessel not using a traffic separation scheme shall avoid it  by as wide a margin as is 
practicable.

A vessel engaged in fishing shall not impede the passage of any vessel following a traffic lane. 
A vessel of less than 20 metres in length or a sailing vessel shall not impede the safe passage 
of a power-driven vessel following a traffic lane.

A  vessel  restricted  in  her  ability  to  manoeuvre  when  engaged  in  an  operation  for  the 
maintenance   of  safety  of  navigation  in  a  traffic  separation  scheme  is  exempted  from 
complying with this Rule to the extent necessary to carry out the operation.

A  vessel  restricted  in  her  ability  to  manoeuvre  when  engaged  in  an  operation  for  the 
laying,  servicing or picking up of a submarine cable, within a traffic separation scheme, is 
exempted from complying with this Rule to the extent necessary to carry out the operation.

Section II - Conduct of vessels in sight of one another

Rule 11

Application

Rules in this Section apply to vessels in sight of one another.

Rule 12

Sailing Vessels

(a)

When two sailing vessels are approaching one another, so as to involve risk of collision, one 
of them shall keep out of the way of the other as follows:

(i)         when each has the wind on a different side, the vessel which has the wind on the port

side shall keep out of the way of the other;

(ii)         when both have the wind on the same side, the vessel which is to windward shall

keep out of the way of the vessel which is to leeward;

(iii)        if a vessel with the wind on the port side sees a vessel to windward and cannot 
	determine with certainty whether the other vessel has the wind on the port or on the

starboard side, she shall keep out of the way of the other.

(b)

For the purposes of this Rule the windward side shall be deemed to be the side opposite to 
that  on which the mainsail is carried or, in the case of a square-rigged vessel, the side 
opposite to that on which the largest fore-and-aft sail is carried.

Rule 13

Overtaking

(a)

(b)

(c)

(d)

Notwithstanding  anything  contained  in  the  Rules  of  Part  B,  Sections  I  and  II,  any  vessel 
overtaking any other shall keep out of the way of the vessel being overtaken.

A vessel shall be deemed to be overtaking when coming up with another vessel from a 
direction  more than 22.5 degrees abaft her beam, that is, in such a position with reference to 
the vessel she is overtaking, that at night she would be able to see only the stern light of that 
vessel but neither of her sidelights.

When a vessel is in any doubt as to whether she is overtaking another, she shall assume that 
this is the case and act accordingly.

Any  subsequent  alteration  of  the  bearing  between  the  two  vessels  shall  not  make  the 
overtaking  vessel a crossing vessel within the meaning of these Rules or relieve her of the 
duty of keeping clear of the overtaken vessel until she is finally past and clear.

Rule 14

Head-on situation

(a)

(b)

(c)

When two power-driven vessels are meeting on reciprocal or nearly reciprocal courses so as 
to  involve risk of collision each shall alter her course to starboard so that each shall pass on the 
port side of the other.

Such a situation shall be deemed to exist when a vessel sees the other ahead or nearly ahead 
and by night she would see the mast head lights of the other in a line or nearly in a line and 
or both sidelights and by day she observes the corresponding aspect of the other vessel.

When a vessel is in any doubt as to whether such a situation exists she shall assume that it 
does exist and act accordingly.

Rule 15

Crossing situation

When two power-driven vessels are crossing so as to involve risk of collision, the vessel which has the 
other on her own starboard side shall keep out of the way and shall, if the circumstances of the case 
admit, avoid crossing ahead of the other vessel.

Rule 16

Action by give-way vessel

Every vessel which is directed to keep out of the way of another vessel shall, so far as possible, take 
early and substantial action to keep well clear.

Rule 17

Action by stand-on vessel

(a)

(i)

Where  one  of  two  vessels  is  to  keep  out  of  the  way  the  other  shall  keep  her 
course and speed.

(iii)

The latter vessel may however take action to avoid collision by her manoeuvre alone, 
as soon as it becomes apparent to her that the vessel required to keep out of the way 
is not taking appropriate action in compliance with these Rules.

(b)

(c)

(d)

When, from any cause, the vessel required to keep her course and speed finds herself so close 
that collision cannot be avoided by the action of the give-way vessel alone, she shall take such 
action as will best aid to avoid collision.

A power-driven vessel which takes action in a crossing situation in accordance with sub- 
paragraph (a)(ii) of this Rule to avoid collision with another power-driven vessel shall, if the 
circumstances of the case admit, not alter course to port for a vessel on her own port side.

This Rule does not relieve the give-way vessel of her obligation to keep out of the way.

Rule 18

Responsibilities between vessels

Except where Rules 9,10 and 13 otherwise require:

(a)

A power-driven vessel underway shall keep out of the way of:

(i) 
(ii) 
(iii) 
(iv)

a vessel not under command;

a vessel restricted in her ability to manoeuvre; 
a vessel engaged in fishing;

a sailing vessel.

(b)

A sailing vessel underway shall keep out of the way of:

(i) 
(ii) 
(iii)

a vessel not under command;

a vessel restricted in her ability to manoeuvre; 
a vessel engaged in fishing.

(c)

A vessel engaged in fishing when underway shall, so far as possible, keep out of the way of:

(d)

(i) 
(ii) 
(i)

a vessel not under command;

a vessel restricted in her ability to manoeuvre.

Any vessel other than a vessel not under command or a vessel restricted in her ability

to  manoeuvre shall, if the circumstances of the case admit, avoid impeding the safe

passage of a vessel constrained by her draught, exhibiting the signals in Rule 28.

(ii)       A vessel constrained by her draught shall navigate with particular caution having full

regard to her special condition.

(e)

A seaplane on the water shall, in general, keep well clear of all vessels and avoid impeding their 
navigation. In circumstances, however, where risk of collision exists, she shall comply with the 
Rules of this Part.

(f)

(i)      A WIG craft shall, when taking off, landing and in flight near the surface, keep well clear

of all other vessels and avoid impeding their navigation;

(ii)     A WIG craft operating on the water surface shall comply with the Rules of this Part as a

power-driven vessel.

Section III - Conduct of vessels in restricted visibility

Rule 19

Conduct of vessels in restricted visibility

(a)

(b)

(c)

(d)

This Rule applies to vessels not in sight of one another when navigating in or near an area 
of restricted visibility.

Every vessel shall proceed at a safe speed adapted to the prevailing circumstances and 
conditions of restricted visibility. A power-driven vessel shall have her engines ready for 
immediate manoeuvre.

Every  vessel  shall  have  due  regard  to  the  prevailing  circumstances  and  conditions  of 
restricted visibility when complying with the Rules of Section I of this Part.

A vessel which detects by radar alone the presence of another vessel shall determine if a 
close-quarters situation is developing and/or risk of collision exists. If so, she shall take 
avoiding action in ample time, provided that when such action consists of an alteration 
of course, so far as possible the following shall be avoided:

(i)

(ii)

an alteration of course to port for a vessel forward of the beam, other than for a 
vessel being overtaken;

an alteration of course towards a vessel abeam or abaft the beam.

(e)

Except where it has been determined that a risk of collision does not exist, every vessel 
which  hears apparently forward of her beam the fog signal of another vessel, or which 
cannot avoid a  close-quarters  situation  with  another  vessel  forward  of  her  beam,  shall 
reduce her speed to the  minimum at which she can be kept on her course. She shall if 
necessary take all her way off and in any event navigate with extreme caution until danger of 
collision is over.

PART C - LIGHTS AND SHAPES

Rule 20

Application

(a)

(b)

(c)

(d)

(e)

Rules in this Part shall be complied with in all weathers.

The Rules concerning lights shall be complied with from sunset to sunrise and during such times 
no other lights shall be exhibited, except such lights as cannot be mistaken for the lights 
specified in these Rules or do not impair their visibility or distinctive character, or interfere 
with the keeping of a proper look-out.

The lights prescribed by these Rules shall, if carried, also be exhibited from sunrise to sunset in 
restricted visibility and may be exhibited in all other circumstances when it is deemed necessary. 
The Rules concerning shapes shall be complied with by day.

The lights and shapes specified in these Rules shall comply with the provisions of Annex I to these 
Regulations.

Rule 21

Definitions

(a)

(b)

(c)

(d)

(e)

(f)

“Masthead light” means a white light placed over the fore and aft centreline of the vessel

showing  an unbroken light over an arc of the horizon of 225 degrees and so fixed as to show 
the light from right ahead to 22.5 degrees abaft the beam on either side of the vessel.

“Sidelights” means a  green light  on  the  starboard side and a  red light on the port  side

each showing an unbroken light over an arc of the horizon of 112.5 degrees and so fixed as to 
show the light from the right ahead to 22.5 degrees abaft the beam on its respective side. In a 
vessel of less than 20 metres in length the sidelights may be combined in one lantern carried on 
the fore and aft centreline of the vessel.

“Sternlight” means a white light placed as nearly as practicable at the stern showing an unbroken

light over an arc of the horizon of 135 degrees and so fixed as to show the light 67.5 degrees 
from right aft on each side of the vessel.

“Towing light” means a yellow light having the same characteristics as the “sternlight” defined in

paragraph (c) of this Rule.

“All-round light” means a light  showing  an  unbroken  light  over  an  arc  of  the  horizon  of 
360 degrees.

“Flashing light” means a light flashing at regular intervals at a frequency of 120 flashes or more 
per minute.

Rule 22

Visibility of lights

The lights prescribed in these Rules shall have an intensity as specified in Section 8 of Annex I to 
these Regulations so as to be visible at the following minimum ranges:

(a)

In vessels of 50 metres or more in length:

- a masthead light, 6 miles;

- a sidelight, 3 miles;

- a stern light, 3 miles;

- a towing light, 3 miles;

- a white, red, green or yellow all-round light, 3 miles.

(b)

In vessels of 12 metres or more in length but less than 50 metres in length:

- a masthead light,  5 miles; except that where the length  of the  vessel is less than 20

metres, 3 miles;

(c)

(d)

- a sidelight, 2 miles;

- a sternlight, 2 miles;

- a towing light, 2 miles;

- a white, red, green or yellow all-round light, 2 miles.

In vessels of less than 12 metres in length:

- a masthead light, 2 miles;

- a sidelight, 1 mile;

- a sternlight, 2 miles;

- a towing light, 2 miles

- a white, red, green or yellow all-round light, 2 miles.

In inconspicuous, partly submerged vessels or objects being towed:

- a white all-round light, 3 miles.

Rule 23

Power-driven vessels underway

(a)

A power-driven vessel underway shall exhibit:

(i) 
(ii)

(iii)

(iv)

a masthead light forward;

a second masthead light abaft of and higher than the forward one; except that a

vessel of less than 50 metres in length shall not be obliged to exhibit such light but 
may do so;

sidelights;

a sternlight.

(b)

(c)

(d)

An air-cushion vessel when operating in the non-displacement mode shall, in addition to 
the lights prescribed in paragraph (a) of this Rule, exhibit an all-round flashing yellow light.

A WIG craft only when taking off, landing and in flight near the surface shall, in addition 
to  the lights  prescribed in  paragraph (a)  of this Rule, exhibit  a high intensity  all-round 
flashing red light.

(i)        A power-driven vessel of less than 12 metres in length may in lieu of the lights

prescribed in paragraph (a) of this Rule exhibit an all-round white light and sidelights;

(ii)         a power-driven vessel of less than 7 metres in length whose maximum speed does 
	not  exceed 7 knots may in lieu of the lights prescribed in paragraph (a) of this Rule

exhibit an all-round white light and shall, if practicable, also exhibit sidelights;

(iii)        the masthead light or all-round white light on a power-driven vessel of less than

12  metres in  length may be displaced  from the  fore  and aft  centre line of the 
vessel  if  centreline  fitting  is  not  practicable,  provided  that  the  sidelights  are 
combined in one lantern which shall be carried on the fore and aft centre line of 
the vessel or located as nearly as practicable in the same fore and aft line as the 
masthead light or the all-round white light.

Rule 24

Towing and pushing

(a)

A power-driven vessel when towing shall exhibit:

(i)         instead of the light prescribed in Rule 23(a)(i) or (a)(ii), two masthead lights in a 
	vertical line. When the length of the tow, measuring from the stern of the towing vessel

to the after end of the tow exceeds 200 metres, three such lights in a vertical line;

(ii)         sidelights;

(iii)        a sternlight;

(iv)        a towing light in a vertical line above the sternlight;

(v)         when the length of the tow exceeds 200 metres, a diamond shape where it can

best be seen.

(b)

(c)

When a pushing vessel and a vessel being pushed ahead are rigidly connected in a composite unit 
they shall be regarded as a power-driven vessel and exhibit the lights prescribed in Rule 23.

A power-driven  vessel  when  pushing  ahead  or  towing  alongside,  except  in  the  case 
of  a composite unit, shall exhibit:

(i)         instead  of  the  light  prescribed  in  Rule  23(a)(i)  or  (a)(ii),  two  masthead  lights  in

a vertical line;

(ii)         sidelights;

(iii)        a sternlight.

(d)

(e)

A power-driven vessel to which paragraph (a) or (c) of this Rule applies shall also comply 
with Rule 23(a) (ii).

A vessel  or  object  being  towed,  other  than  those  mentioned  in  paragraph  (g)  of  this 
Rule, shall exhibit:

(i) 
(ii) 
(iii)

sidelights;

a sternlight;

when the length of the tow exceeds 200 metres, a diamond shape where it can

best be seen.

(f)

Provided that any number of vessels being towed alongside or pushed in a group shall be 
lighted as one vessel,

(i)         a vessel being pushed ahead, not being part of a composite unit, shall exhibit at

the forward end sidelights;

(ii)         a vessel being towed alongside shall  exhibit  a  sternlight  and  at  the  forward

end, sidelights.

(g)

An inconspicuous, partly submerged vessel or object, or combination of such vessels or 
objects being towed, shall exhibit:

(i)         if it is less than 25 metres in breadth, one all-round white light at or near the forward 
	end and one at or near the after end except that dracones need not exhibit a light at

or near the forward end;

(ii)         if it is 25 metres or more in breadth, two additional all-round white lights at or near

the extremities of its breadth;

(iii)        if it  exceeds 100 metres in  length, additional all-round white lights between the

lights prescribed in sub-paragraphs (i) and (ii) so that the distance between the lights 
shall not exceed 100 metres;

(iv)        a diamond shape at or near the aftermost extremity of the last vessel or object being 
	towed  and if the length of the tow exceeds 200 metres an additional diamond shape

where it can best be seen and located as far forward as is practicable.

(h)

(i)

Where from any sufficient cause it is impracticable for a vessel or object being towed to exhibit 
the lights or shapes prescribed in paragraph (e) or (g) of this Rule, all possible measures shall 
be taken  to light the vessel or object towed or at least to indicate the presence of such 
vessel or object.

Where from any sufficient cause it is impracticable for a vessel not normally engaged in 
towing  operations to display the lights prescribed in paragraph (a) or (c) of this Rule, such 
vessel shall not be required to exhibit those lights when engaged in towing another vessel in 
distress or otherwise in need of assistance. All possible measures shall be taken to indicate the 
nature of the  relationship   between  the  towing  vessel  and  the  vessel  being  towed  as 
authorized by Rule 36, in particular by illuminating the towline.

Rule 25

Sailing vessels underway and vessels under oars

(a)

A sailing vessel underway shall exhibit:

(i) 
(ii)

sidelights;

a sternlight.

(b)

(c)

In a sailing vessel of less than 20 metres in length the lights prescribed in paragraph (a) of this 
Rule may be combined in one lantern carried at or near the top of the mast where it can best 
be seen.

A sailing vessel underway may, in addition to the lights prescribed in paragraph (a) of this Rule, 
exhibit at or near the top of the mast, where they can best be seen, two all-round lights in a 
vertical line, the upper being red and the lower green, but these lights shall not be exhibited in 
conjunction with the combined lantern permitted by paragraph (b) of this Rule.

(d)

(i)      A sailing vessel of less than 7 metres in length shall, if practicable, exhibit the lights

prescribed in paragraph (a) or (b) of this Rule, but if she does not, she shall have ready 
at hand an electric torch or lighted lantern showing a white light which shall be exhibited 
in sufficient time to prevent collision.

(ii)      A vessel under oars may exhibit the lights prescribed in this Rule for sailing vessels, but 
	if she does not, she shall have ready at hand an electric torch or lighted lantern

showing a white light which shall be exhibited in sufficient time to prevent collision.

(e)

A vessel proceeding under sail when also being propelled by machinery shall exhibit forward 
where it can best be seen a conical shape, apex downwards.

Rule 26 
Fishing Vessels

(a)

(b)

A vessel engaged in fishing, whether underway or at anchor, shall exhibit only the lights 
and shapes prescribed in this Rule.

A vessel when engaged in trawling, by which is meant the dragging through the water of a 
dredge net or other apparatus used as a fishing appliance, shall exhibit:

(i)         two all-round lights in a vertical line, the upper being green and the lower white, or

a shape consisting of two cones with their apexes together in a vertical line one

above the other;

(ii)         a masthead light abaft of and higher than the all-round green light; a vessel of less

than 50 metres in length shall not be obliged to exhibit such a light but may do so; 
(iii)        when making way through the water, in addition to the lights prescribed in this

paragraph, sidelights and a stern light.

(c)

A vessel engaged in fishing, other than trawling, shall exhibit:

(i)         two all-round lights in a vertical line, the upper being red and the lower white, or a

shape consisting of two cones with apexes together in a vertical line one above the

other;

(ii)         when there is outlying gear extending more than 150 metres horizontally from the

vessel, an all- round white light or a cone apex upwards in the direction of the gear; 
(iii)        when making way through the water, in addition to the lights prescribed in this

paragraph, sidelights and a sternlight.

(d)

(e)

The additional signals described in Annex II to these Regulations apply to a vessel engaged 
in fishing in close proximity to other vessels engaged in fishing.

A vessel when not engaged in fishing shall not exhibit the lights or shapes prescribed in this 
Rule, but only those prescribed for a vessel of her length.

Rule 27

Vessels not under command or restricted in their ability to manoeuvre

(a)

A vessel not under command shall exhibit:

(i) 
(ii) 
(iii)

two all-round red lights in a vertical line where they can best be seen;

two balls or similar shapes in a vertical line where they can best be seen;

when  making  way  through  the  water,  in  addition  to  the  lights  prescribed  in

this paragraph, sidelights and a stern light.

(b)

A  vessel restricted in her ability to manoeuvre, except a vessel engaged in mine-clearance 
operations, shall exhibit:

(i)         three all-round lights in a vertical line where they can best be seen. The highest and

lowest of these lights shall be red and the middle light shall be white;

(ii)         three shapes in a vertical line where they can best be seen. The highest and lowest of

these shapes shall be balls and the middle one a diamond;

(iii)        when  making  way  through  the  water,  a  masthead  light  or  lights,  sidelights  and

a  sternlight, in addition to the lights prescribed in sub-paragraph (i);

(iv)        when at anchor, in addition to the lights or shapes prescribed in sub-paragraphs (i)

and (ii), the light, lights or shape prescribed in Rule 30.

(c)

(d)

A power-driven vessel engaged in a towing operation such as severely restricts the towing 
vessel and her tow in their ability to deviate from their course shall, in addition to the lights 
or shapes prescribed in Rule 24(a), exhibit the lights or shapes prescribed in sub-paragraphs 
(b)(i) and (ii) of this Rule.

A  vessel engaged in dredging or underwater operations, when restricted in her ability to

manoeuvre, shall exhibit the lights and shapes prescribed in sub-paragraphs (b) (i), (ii) and (iii) 
of this Rule and shall in addition, when an obstruction exists, exhibit:

(i)         two all-round red lights or two balls in a vertical line to indicate the side on which

the obstruction exists;

(ii)         two all-round green lights or two diamonds in a vertical line to indicate the side on

which another vessel may pass;

(iii)        when at anchor, the lights or shapes prescribed in this paragraph instead of the lights

or shape prescribed in Rule 30.

(e)

Whenever the size of a vessel engaged in diving operations makes it impracticable to exhibit 
all lights and shapes prescribed in paragraph (d) of this Rule, the following shall be exhibited: 
(i)         three all-round lights in a vertical line where they can best be seen. The highest and

(ii)

lowest of these lights shall be red and the middle light shall be white;

a rigid replica of the International Code flag “A” not less than 1 metre in height.

Measures shall be taken to ensure its all-round visibility.

(f)

(g)

(h)

A vessel engaged in mine-clearance operations shall in addition to the lights prescribed 
for a  power-driven vessel in Rule 23 or to the lights or shape prescribed for a vessel at 
anchor in Rule 30 as appropriate, exhibit three all-round green lights or three balls. One of 
these lights or shapes  shall be exhibited near the foremast head and one at each end of 
the fore yard. These lights or  shapes indicate that it is dangerous for another vessel to 
approach within 1000 metres of the mine clearance vessel.

Vessels of less than 12 metres in length, except those engaged in diving operations, shall 
not be required to exhibit the lights and shapes prescribed in this Rule.

The  signals  prescribed  in  this  Rule  are  not  signals  of  vessels  in  distress  and  requiring 
assistance. Such signals are contained in Annex IV to these Regulations.

Rule 28

Vessels constrained by their draught

A vessel constrained by her draught may, in addition to the lights prescribed for power-driven vessels 
in Rule 23, exhibit where they can best be seen three all-round red lights in a vertical line, or a cylinder.

Rule 29

Pilot vessels

(a)

A vessel engaged on pilotage duty shall exhibit:

(i)         at or near the masthead, two all-round lights in a vertical line, the upper being white

and the lower red;

(ii)         when underway, in addition, sidelights and a sternlight;

(iii)        when at anchor, in addition to the lights prescribed in sub-paragraph (i), the light,

lights or shape prescribed in Rule 30 for vessels at anchor.

(b)

A pilot vessel when not engaged on pilotage duty shall exhibit the lights or shapes prescribed 
for a similar vessel of her length.

Rule 30

Anchored vessels and vessels aground

(a)

A vessel at anchor shall exhibit where it can best be seen:

(i) 
(ii)

in the fore part, an all-round white light or one ball;

at or near the stern and at a lower level than the light prescribed in sub-paragraph (i),

an all-round white light.

(b)

(c)

(d)

A vessel of less than 50 metres in length may exhibit an all-round white light where it can best 
be seen instead of the lights prescribed in paragraph (a) of this Rule.

A vessel at anchor may, and a vessel of 100 metres and more in length shall, also use the 
available working or equivalent lights to illuminate her decks.

A vessel aground shall exhibit the lights prescribed in paragraph (a) or (b) of this Rule and 
in addition, where they can best be seen:

(i) 
(ii)

two all-round red lights in a vertical line; 
three balls in a vertical line.

(e)

(f)

A vessel of less than 7 metres in length, when at anchor, not in or near a narrow channel, 
fairway  or anchorage, or where other vessels normally navigate, shall not be required to 
exhibit the lights or shape prescribed in paragraphs (a) and (b) of this Rule.

A vessel of less than 12 metres in length, when aground, shall not be required to exhibit the 
lights or shapes prescribed in sub-paragraphs (d) (i) and (ii) of this Rule.

Rule 31

Seaplanes

Where  it  is  impracticable  for  a  seaplane  or  a  WIG  craft  to  exhibit  lights  and  shapes  of  the 
characteristics or in the positions prescribed in the Rules of this Part she shall exhibit lights and shapes 
as closely similar in characteristics and position as is possible.

PART D - SOUND AND LIGHT SIGNALS

Rule 32

Definitions

(a)

(b)

(c)

The  word  “whistle”  means  any  sound  signalling  appliance  capable  of  producing  the

prescribed blasts and which complies with the specifications in Annex III to these Regulations.

The term “short blast” means a blast of about one second’s duration.

The term “prolonged blast” means a blast of from four to six seconds’ duration.

Rule 33

Equipment for sound signals

(a)

(b)

A vessel of 12 metres or more in length shall be provided with a whistle, a vessel of 20 
metres or more in length shall be provided with a bell in addition to a whistle, and a vessel of 
100 metres or  more in length shall, in addition, be provided with a gong, the tone and 
sound of which cannot  be confused with that of the bell. The whistle, bell and gong shall 
comply with the specifications in Annex III to these Regulations. The bell or gong or both 
may be replaced by other equipment  having the same respective sound characteristics, 
provided that manual sounding of the prescribed signals shall always be possible.

A  vessel  of  less  than  12  metres  in  length  shall  not  be  obliged  to  carry  the  sound 
signalling appliances prescribed in paragraph (a) of this Rule but if she does not, she shall be 
provided with some other means of making an efficient sound signal.

Rule 34

Manoeuvring and warning signals

(a)

(b)

When vessels are in sight of one another, a power-driven vessel underway, when manoeuvring 
as  authorized or required by these Rules, shall indicate that manoeuvre by the following 
signals on her whistle:

- one short blast to mean “I am altering my course to starboard”;

- two short blasts to mean “I am altering my course to port”;

- three short blasts to mean “I am operating astern propulsion”.

Any vessel may supplement the whistle signals prescribed in paragraph (a) of this Rule by

light signals, repeated as appropriate, whilst the manoeuvre is being carried out:

(i)         these light signals shall have the following significance

- one flash to mean “I am altering my course to starboard”;

- two flashes to mean “I am altering my course to port”;

- three flashes to mean “I am operating astern propulsion”;

(ii)         the duration of each flash shall be about one second, the interval between flashes

shall be about one second, and the interval between successive signals shall be not 
less than ten seconds;

(iii)        the  light  used  for  this  signal  shall,  if  fitted,  be  an  all-round  white  light,  visible 
	at a minimum range of 5 miles, and shall  comply  with  the  provisions  of Annex  I

to these Regulations.

(c)

When in sight of one another in a narrow channel or fairway:

(i)         a vessel intending to overtake another shall in compliance with Rule 9(e)(i) indicate

her intention by the following signals on her whistle:

- two prolonged blasts followed by one short blast to mean “I intend to overtake you on

your starboard side”;

- two prolonged blasts followed by two short blasts to mean “I intend to overtake you on

your port side”.

(ii)         the vessel about to be overtaken when acting in accordance with Rule 9(e)(i) shall

indicate her agreement by the following signal on her whistle:

(d)

(e)

(f)

- one prolonged, one short, one prolonged and one short blast, in that order.

When vessels in sight of one another are approaching each other and from any cause either 
vessel  fails to understand the intentions or actions of the other, or is in doubt whether 
sufficient action  is being taken by the other to avoid collision, the vessel in doubt shall 
immediately indicate such  doubt  by  giving  at  least  five  short  and  rapid  blasts  on  the 
whistle. Such signal may be supplemented by a light signal of at least five short and rapid 
flashes.

A vessel nearing a bend or an area of a channel or fairway where other vessels may be 
obscured by an intervening obstruction shall sound one prolonged blast. Such signal shall be 
answered with  a prolonged blast by any approaching vessel that may be within hearing 
around the bend or behind the intervening obstruction.

If whistles are fitted on a vessel at a distance apart of more than 100 metres, one whistle only 
shall be used for giving manoeuvring and warning signals.

Rule 35

Sound signals in restricted visibility

In or near an area of restricted visibility, whether by day or night, the signals prescribed in this Rule shall 
be used as follows:

(a)        A power-driven vessel making way through the water shall sound at intervals of not more

than 2 minutes one prolonged blast.

(b)        A power-driven vessel underway but stopped and making no way through the water shall

sound at intervals of not more than 2 minutes two prolonged blasts in succession with an 
interval of about 2 seconds between them.

(c)         A vessel not  under command, a vessel restricted in her ability to manoeuvre, a vessel 
	constrained  by her  draught,  a  sailing  vessel,  a vessel  engaged  in  fishing  and  a  vessel 
	engaged  in  towing  or  pushing another vessel shall, instead of the signals prescribed in 
	paragraphs (a) or (b) of this Rule, sound at intervals of not more than 2 minutes three blasts

in succession, namely one prolonged followed by two short blasts.

(d)        A vessel  engaged  in  fishing,  when  at  anchor,  and  a  vessel  restricted  in  her  ability  to

manoeuvre when carrying out her work at anchor, shall instead of the signals prescribed in 
paragraph (g) of this Rule sound the signal prescribed in paragraph (c) of this Rule.

(e)        A vessel towed or if more than one vessel is towed the last vessel of the tow, if manned, 
	shall at  intervals of not more than 2 minutes sound four blasts in succession, namely one 
	prolonged  followed  by  three  short  blasts.  When  practicable,  this  signal  shall  be  made

immediately after the signal made by the towing vessel.

(f)         When a pushing vessel and a vessel being pushed ahead are rigidly connected in a composite

unit they shall be regarded as a power-driven vessel and shall give the signals prescribed in 
paragraphs (a) or (b) of this Rule.

(g)        A vessel at anchor shall at intervals of not more than one minute ring the bell rapidly for 
	about 5 seconds. In a vessel of 100 metres or more in length the bell shall be sounded in the 
	forepart of the  vessel and immediately after the  ringing  of the bell the  gong shall be 
	sounded rapidly for about 5 seconds in the after part of the vessel. A vessel at anchor may in 
	addition sound three blasts in  succession, namely one short, one prolonged and one short

blast, to give warning of her position  and of the possibility of collision to an approaching

vessel.

(h)        A vessel aground shall give the bell signal and if required the gong signal prescribed in

paragraph

(g)  of  this  Rule  and  shall,  in  addition,  give  three  separate  and  distinct  strokes  on  the 
bell immediately before and after the rapid ringing of the bell. A vessel aground may in 
addition sound an appropriate whistle signal.

(i)

(j)

(k)

A vessel of 12 metres or more but less than 20 metres in length shall not be obliged to give 
the bell signals prescribed in paragraphs (g) and (h) of this Rule. However, if she does not, 
she shall make some other efficient sound signal at intervals of not more than 2 minutes.

A vessel of less than 12 metres in length shall not be obliged to give the above-mentioned 
signals but, if she does not, shall make some other efficient sound signal at intervals of not 
more than 2 minutes.

A pilot vessel when engaged on pilotage duty may in addition to the signals prescribed 
in paragraphs  (a),(b)  or  (g)  of  this  Rule  sound   an   identity   signal   consisting   of 
four  short blasts.

Rule 36

Signals to attract attention

If necessary to attract the attention of another vessel any vessel may make light or sound signals 
that cannot be mistaken for any signal authorised elsewhere in these Rules, or may direct the beam 
of her searchlight in the direction of the danger, in such a way as not to embarrass any vessel. Any light 
to attract  the attention of another vessel shall be such that it cannot be mistaken for any aid to 
navigation. For the purpose of this Rule the use of high intensity intermittent or revolving lights, such 
as strobe lights, shall be avoided.

Rule 37

Distress signals

When a vessel is in distress and requires assistance she shall use or exhibit the signals described in 
Annex IV to these Regulations.

PART E - EXEMPTIONS

Rule 38

Exemptions

Any  vessel  (or  class  of  vessels)  provided  that  she  complies  with  the  requirements  of  the 
International  Regulations for Preventing Collisions at Sea, 1960 (a), the keel of which is laid or which 
is at a corresponding stage of construction before the entry into force of these Regulations may be 
exempted from compliance therewith as follows:

(a)  The installation of lights with ranges prescribed in Rule 22, until 4 years after the date of entry

into force of these Regulations.

(b)  The installation of lights with colour specifications as prescribed in Section 7 of Annex I to these

Regulations, until 4 years after the date of entry into force of these Regulations.

(c)  The repositioning of lights as a result of conversion from Imperial to metric units and rounding

off measurement figures, permanent exemption.

(d)

(i)         The repositioning of masthead lights on vessels of less than 150 metres in length,

resulting  from  the prescriptions  of  Section  3(a)  of Annex  I  to  these  Regulations, 
permanent exemption.

(ii)         The repositioning of masthead lights on vessels of 150 metres or more in length, 
	resulting  from the prescriptions of Section 3(a) of Annex I to these Regulations, until 9

years after the date of entry into force of these Regulations.

(e)  The repositioning of masthead lights resulting from the prescriptions of Section 2(b) of Annex I

to these Regulations, until 9 years after the date of entry into force of these Regulations.

(f)   The repositioning of sidelights resulting from the prescriptions of Sections 2(g) and 3(b) of 
	Annex   I  to  these  Regulations,  until  9  years  after  the  date  of  entry  into  force  of  these

Regulations.

(g)  The requirements for sound signal appliances prescribed in Annex III to these Regulations, until

9 years after the date of entry into force of these Regulations.

(h)  The repositioning of all-round lights resulting from the prescription of Section 9(b) of Annex I

to  these Regulations, permanent exemption.

PART F – VERIFICATION OF COMPLIANCE WITH THE PROVISIONS OF THE 
	CONVENTION

Rule 39

Definitions

(a)  Audit means a systematic, independent and documented process for obtaining audit evidence and

evaluating it objectively to determine the extent to which audit criteria are fulfilled.

(b)  Audit Scheme means the IMO Member State Audit Scheme established by the Organization and

taking into account the guidelines developed by the Organization.

(c)  Code for Implementation means the IMO Instruments Implementation Code (III Code) adopted by

the Organization by resolution A.1070(28).

(d)  Audit Standard means the Code for Implementation.

Rule 40

Application

Contracting Parties shall use the provisions of the Code for Implementation in the execution of their 
obligations and responsibilities contained in the present Convention.

Rule 41

Verification of compliance

(a)  Every Contracting Party shall be subject to periodic audits by the Organization in accordance with

the audit standard to verify compliance with and implementation of the present Convention.

(b)  The Secretary-General of the Organization shall have responsibility for administering the Audit

Scheme, based on the guidelines developed by the Organization.

(c)  Every Contracting Party shall have responsibility for facilitating the conduct of the audit and 
	implementation of a programme of actions to address the findings, based on the guidelines

developed by the Organization.

(d)  Audit of all Contracting Parties shall be:

(i)   based on an overall schedule developed by the Secretary General of the Organization, taking

into account the guidelines developed by the Organization; and

(ii)  conducted  at  periodic  intervals,  taking  into  account  the  guidelines  developed  by  the

Organization.

ANNEX I

Positioning and Technical Details of Lights and Shapes

1.

Definition

The term “height above the hull” means height above the uppermost continuous deck. This height

shall be measured from the position vertically beneath the location of the light.

2.

Vertical positioning and spacing of lights

(a)

On a power-driven vessel of 20  metres or  more in  length the masthead lights shall be 
placed as follows:

(i)         the forward masthead light, or if only one masthead light is carried, then that light, at 
	a  height above the hull of not less than 6 metres, and, if the breadth of the vessel 
	exceeds 6  metres, then at a height above the hull not less than such breadth, so 
	however that the light need not be placed at a greater height above the hull than 12

metres;

(ii)         when two  masthead lights  are carried the  after one shall be at  least 4.5  metres

vertically higher than the forward one.

(b)

(c)

(d)

(e)

The vertical separation of masthead lights of power-driven vessels shall be such that in all 
normal conditions of trim the after light will be seen over and separate from the forward light at 
a distance of 1,000 metres from the stem when viewed from sea-level.

The masthead light of a power-driven vessel of 12 metres but less than 20 metres in length 
shall be placed at a height above the gunwale of not less than 2.5 metres.

A power-driven vessel of less than 12 metres in length may carry the uppermost light at a 
height of less than 2.5 metres above the gunwale. When however a masthead light is carried 
in addition  to sidelights and a sternlight or the all-round light prescribed in Rule 23(c)(i) is 
carried in addition  to sidelights, then such masthead light or all-round light shall be carried 
at least 1 metre higher than the sidelights.

One of the two or three masthead lights prescribed for a power-driven vessel when engaged 
in  towing or pushing another vessel shall be placed in  the  same position as either the 
forward masthead light or the after masthead light; provided that, if carried on the aftermast, 
the lowest  after masthead light shall be at least 4.5 metres vertically higher than the forward 
masthead light.

(f)

(i)        The masthead light or lights prescribed in Rule 23(a) shall be so placed as to be above 
	and clear of all other lights and obstructions except as described in sub-paragraph (ii).

(ii)       When it is impracticable to carry the all-round lights prescribed by Rule 27(b)(i) or Rule 
	28 below the masthead lights, they may be carried above the after masthead light(s)

or  vertically  in  between   the  forward  masthead  light(s)  and  the  after  masthead

light(s)  provided  that  in  the  latter  case  the  requirement  of  Section  3(c)  of  this

Annex shall be complied with.

(g)

(h)

(i)

The sidelights of a power-driven vessel shall be placed at a height above the hull not greater 
than three-quarters of that of the forward masthead light. They shall not be so low as to be 
interfered  with by deck lights.

The sidelights, if in a combined lantern and carried on a power-driven vessel of less than 
20 metres in length, shall be placed not less than 1 metre below the masthead light.

When the Rules prescribe two or three lights to be carried in a vertical line, they shall be 
spaced as follows:

(i)         on a vessel of 20 metres in length or more such lights shall be spaced not less

than 2 metres apart, and the lowest of these lights shall, except where a towing light is 
required, be placed at a height of not less than 4 metres above the hull;

(ii)         on a vessel of less than 20 metres in length such lights shall be spaced not less than 1 
	metre  apart  and  the  lowest  of  these  lights  shall,  except  where  a  towing  light  is

required, be placed at a height of not less than 2 metres above the gunwale;

(j)

(k)

(iii)        when three lights are carried they shall be equally spaced.

The lower of the two all-round lights prescribed for a vessel when engaged in fishing shall be 
at  a height above the sidelights not less than twice the distance between the two vertical 
lights.

The forward anchor light prescribed in Rule 30(a)(i), when two are carried, shall not be less 
than 4.5 metres above the after one. On a vessel of 50 metres or more in length this forward 
anchor light shall be placed at a height of not less than 6 metres above the hull.

3.

Horizontal positioning and spacing of lights

(a)

(b)

(c)

When  two  masthead  lights  are  prescribed  for  a  power-driven  vessel,  the  horizontal 
distance  between them shall not be less than one-half of the length of the vessel but need not 
be more than 100 metres. The forward light shall be placed not more than one-quarter of the 
length of the vessel from the stem.

On a power-driven vessel of 20 metres or more in length the sidelights shall not be placed in 
front of the forward masthead lights. They shall be placed at or near the side of the vessel. 
When the  lights prescribed in  Rule 27(b)(i) or  Rule 28 are placed vertically between the 
forward  masthead  light(s)  and  the  after  masthead  light(s) these  all-round  lights  shall be 
placed at a horizontal distance of not less than 2 metres from the fore and aft centreline of

(d)

4.

(a)

(b)

the vessels in the athwartship direction.

When only one masthead light is prescribed for a power-driven vessel, this light shall be 
exhibited forward of amidships; except that a vessel of less than 20 metres in length need not 
exhibit this light forward of amidships but shall exhibit it as far forward as is practicable.

Details of location of direction – indicating lights for fishing vessels, dredgers and vessels

engaged in underwater operations

The light indicating the direction of the outlying gear from a vessel engaged in  fishing 
	as  prescribed in Rule 26(c)(ii). Shall be placed at a horizontal distance of not less than 2 
	metres and  not more than 6 metres away from the two all-round red and white lights. 
	This light shall be placed not higher than the all-round white light prescribed in Rule 26(c)(i) 
	and not lower than the sidelights.

The lights and shapes on a vessel engaged in dredging or underwater operations to indicate 
	the obstructed side and or the side on which it is safe to pass, as prescribed in Rule 27(d)(i) 
	and (ii),  shall be placed at the maximum practical horizontal distance, but in no case less 
	than 2 metres,  from the lights or shapes prescribed in Rule 27(b)(i) and (ii). In no case shall 
	the upper of these lights or shapes be at a greater height than the lower of the three lights 
	or shapes prescribed in Rule 27(b)(i) and (ii).

5.

Screens for sidelights

The sidelights of vessels of 20 metres or more in length shall be fitted with inboard screens painted 
matt  black, and meeting the requirements of Section 9 of this Annex. On vessels of less than 20 
metres in length the sidelights, if necessary to meet the requirements of Section 9 of this Annex, shall 
be fitted with inboard  matt black screens. With a combined lantern, using a single vertical filament 
and a very narrow division between the green and red sections, external screens need not be fitted.

6.

Shapes

(a)

Shapes shall be black and of the following sizes:

(i) 
(ii)

(iii)

(iv)

a ball shall have a diameter of not less than 0.6 metre;

a cone shall have a base diameter of not less than 0.6 metre and a height equal

to its diameter;

a cylinder shall have a diameter of at least 0.6 metre and a height of twice its diameter 
a  diamond  shape  shall  consist  of  two  cones  as  defined  in  (ii)  above  having  a 
common base.

(b)

(c)

The vertical distance between shapes shall be at least 1.5 metres.

In a vessel of less than 20 metres in length shapes of lesser dimensions but commensurate 
with  the size of the vessel may be used and the distance apart may be correspondingly 
reduced.

7.

Colour specification of lights

The chromaticity of all navigation lights shall conform to the following standards, which lie within 
the  boundaries  of  the  area  of  the  diagram  specified  for  each  colour  by  the  International 
Commission on Illumination (CIE).

The boundaries of the area for each colour are given by indicating the corner co-ordinates, which 
are as follows:

(i)

White

x

0.525

0.525

0.310

0.310

0.443

y

0.382

0.440

0.348

0.283

0.382

(ii)

Green

x

0.028

0.009

0.300

0.203

y

0.385

0.723

0.511

0.356

(iii)

Red

x

0.680

0.660

0.735

0.721

y

0.320

0.320

0.265

0.259

(iv)

Yellow

x

0.612

0.618

0.575

0.575

y

0.382

0.382

0.425

0.406

8.

Intensity of lights

(a)

The minimum luminous intensity of lights shall be calculated by using

I = 3.43 x 106 x T x D2 x K–D

Where I

T

D

K

is luminous intensity in candelas under service conditions,

is threshold factor 2 x 10-7  lux,

is range of visibility (luminous range) of the light in nautical miles, 
is atmospheric transmissivity.

For  prescribed  lights  the  value  of  K  shall  be  0.8,  corresponding  to  a  meteorological  visibility 
of approximately 13 nautical miles.

(b)

A selection of figures derived from the formula is given in the following table:

Range of visibility (luminous

Luminous intensity of light in

range) of light in nautical miles 
	D

candelas for K=0.8 
	I

1

0.9

2

4.3

3

12

4

27

5

52

6

94

Note:  The  maximum  luminous  intensity  of  navigation  lights  should  be  limited  to  avoid  undue 
glare. This shall not be achieved by a variable control of the luminous intensity.

9.

Horizontal sectors

(a)

(b)

(i)

(ii)

(i)

(ii)

In the forward direction, sidelights as fitted on the vessel shall show the minimum 
required intensities. The intensities shall decrease to reach practical cut-off between 
1 degree and 3 degrees outside the prescribed sectors.

For stern lights and masthead lights at 22.5 degrees abaft the beam for sidelights, 
the  minimum required intensities shall be maintained over the arc of the horizon 
up to 5 degrees within the limits of the sectors prescribed in Rule 21. From 5 degrees 
within the  prescribed sectors the intensity may decrease by 50 per cent up to the 
prescribed limits: it  shall decrease steadily to reach practical cut-off at not more than 
5 degrees outside the prescribed sectors.

All-round lights shall be so located as not to be obscured by masts, topmasts or 
structures   within  angular  sectors  of  more  than  6  degrees,  except  anchor  lights 
prescribed in Rule 30, which need not be placed at an impracticable height above the 
hull.

If it is impracticable to comply with paragraph (b) (i) of this section by exhibiting only 
one  all-round light, two all-round lights shall be used suitably positioned or screened

so that they appear, as far as practicable, as one light at a distance of one mile.

10.

Vertical sectors

(a)

The  vertical  sectors  of  electric  lights  as  fitted,  with  the  exception  of  lights  on  sailing 
vessels underway shall ensure that:

(i)         at least the required minimum intensity is maintained at all angles from 5 degrees

above to 5 degrees below the horizontal;

(ii)         at  least  60  per  cent  of  the  required  minimum  intensity  is  maintained  from  7.5

degrees  above to 7.5 degrees below the horizontal.

(b)

In  the  case  of  sailing  vessels  underway  the  vertical  sectors  of  electric  lights  as  fitted 
shall ensure that:

(i)         at least the required minimum intensity is maintained at all angles from 5 degrees

above to 5 degrees below the horizontal;

(ii)         at  least  50  per  cent  of  the  required  minimum  intensity  is  maintained  from  25

degrees above to 25 degrees below the horizontal.

(c)

In the case of lights other than electric these specifications shall be met as closely as possible.

11.

Intensity of non-electric lights

Non-electric lights shall so far as practicable comply with the minimum intensities, as specified in the 
table given in Section 8 of this Annex.

12.

Manoeuvring light

Notwithstanding the provisions of paragraph 2(f) of this Annex the manoeuvring light described in 
Rule  34(b) shall be placed in the same fore and aft vertical plane as the masthead light or lights 
and, where  practicable, at a minimum height of 2 metres vertically above the forward masthead 
light, provided that  it shall be carried not less than 2 metres vertically above or below the after 
masthead light. On a vessel where only one masthead light is carried the manoeuvring light, if fitted, 
shall be carried where it can best  be seen, not less than 2 metres vertically apart from the masthead 
light.

13.

High Speed Craft*

(a)

(b)

The masthead light of high-speed craft may be placed at a height related to the breadth of 
the lower than that prescribed in paragraph 2(a)(i) of this annex, provided that the base angle 
of the  isosceles triangles formed by the sidelights and masthead light, when seen in end

elevation, is not less than 27o.

On high-speed craft of 50 metres or more in length, the vertical separation between foremast 
and  mainmast  light  of  4.5  metres  required  by  paragraph  2(a)(ii)  of  this  annex  may  be 
modified  provided that such distance shall not be less than the value determined by the 
following formula:

Y =

(a + 17ψ)

1000

C + 2

Where:

Y

A

ψ

C

is the height of the mainmast light above the foremast light in metres;

is the height of the foremast light above the water surface in service condition 
in metres;

is the trim in service condition in degrees;

is the horizontal separation of masthead lights in metres.

* Refer to the International Code of Safety for High-Speed Craft, 1994 and the International Code of

Safety for High-Speed Craft, 2000.

14.

Approval

The construction of lights and shapes and the installation of lights on board the vessel shall be to 
the satisfaction of the appropriate authority of the State whose flag the vessel is entitled to fly.

ANNEX II

Additional signals for fishing vessels fishing in close proximity

1.

General

The lights mentioned herein shall, if exhibited in pursuance of Rule 26(d), be placed where they can 
best  be seen. They shall be at least 0.9 metre apart but at a lower level than lights prescribed in 
Rule 26(b)(i) and (c)(i). The lights shall be visible all round the horizon at a distance of at least 1 mile 
but at a lesser distance than the lights prescribed by these Rules for fishing vessels.

2.

Signals for trawlers

(a)

Vessels of 20 metres or more in length when engaged in trawling, whether using demersal 
or pelagic gear, shall exhibit:

(i) 
(ii) 
(iii)

when shooting their nets, two white lights in a vertical line;

when hauling their nets, one white light over one red light in a vertical line; 
when the net has come fast upon an obstruction, two red lights in a vertical line.

(b)

Each vessel of 20 metres or more in length engaged in pair trawling shall exhibit:

(i)

(ii)

(c)

by night, a searchlight directed forward and in the direction of the other vessel of the

pair;

when  shooting  or  hauling  their  nets  or  when the  nets  have  come  fast  upon  an 
obstruction, the lights prescribed in 2(a) above.

A vessel of less than 20 metres in length engaged in trawling, whether using demersal 
or  pelagic gear or  engaged in  pair trawling, may exhibit the lights prescribed in 
paragraphs (a) or (b) of this Section, as appropriate.

3.

Signals for purse seiners

Vessels engaged in fishing with purse seine gear may exhibit two yellow lights in a vertical line. 
These lights shall flash alternately every second and with equal light and occultation duration. These 
lights may be exhibited only when the vessel is hampered by its fishing gear.

| Length of vessel in metres | 1/3rd-octave band level at 1 metre in dB referred to 2x10-5N/m2 | Audibility range in nautical miles |
| --- | --- | --- |
| 200 or more | 143 | 2 |
| 75 but less than 200 | 138 | 1.5 |

ANNEX III

Technical details of sound signal appliances

1.

Whistles

(a)

Frequencies and range of audibility

The fundamental frequency of the signal shall lie within the range 70 - 700 Hz. The range of audibility 
of  the  signal  from a  whistle  shall be determined by  those  frequencies, which may include  the 
fundamental  and/or one or more higher frequencies, which lie within the range 180 - 700 Hz (+/-1%) 
for a vessel of 20 metres or more in length, or 180-2100Hz (+/-1%) for a vessel of less than 20 metres 
in length and which provide the sound pressure levels specified in paragraph l(c) below.

(b)

Limits of fundamental frequencies

To  ensure a wide variety of whistle characteristics, the fundamental frequency of a whistle shall 
be between the following limits:

(i)

(ii)

(iii)

(c)

70 - 200 Hz, for a vessel 200 metres or more in length;

130 - 350 Hz, for a vessel 75 metres but less than 200 metres in length; 
250 - 700 Hz, for a vessel less than 75 metres in length.

Sound signal intensity and range of audibility

A whistle fitted in a vessel shall provide, in the direction of maximum intensity of the whistle and at 
a  distance of 1 metre from it, a sound pressure level in at least one 1/3rd-octave band within the 
range of frequencies 180 - 700 Hz (+/-1%) for a vessel of 20 metres or more in length, or 180-2100Hz 
(+/-1%) for a vessel of less than 20 metres in length, of not less than the appropriate figure given in 
the table below.

| 20 but less than 75 | 130 | 1 |
| --- | --- | --- |
| Less than 20 | 120 * | 0.5 |
|  | 115 † |  |
|  | 111 ‡ |  |

* When the measured frequencies lie within the range 180-450Hz

† When the measured frequencies lie within the range 450-800Hz 
‡ When the measured frequencies lie within the range 800-2100Hz

The range of audibility in the table above is for information and is approximately the range at which 
a  whistle may be heard on its forward axis with 90 per cent probability in conditions of still air on 
board a vessel having average background noise level at the listening posts (taken to be 68 dB in the 
octave band centered on 250 Hz and 63 dB in the octave band centered on 500Hz.

In practice the range at which a whistle may be heard is extremely variable and depends critically 
on weather conditions; the values given can be regarded as typical but under conditions of strong 
wind or high ambient noise level at the listening post the range may be much reduced.

(d)

Directional Properties

The sound pressure level of a directional whistle shall be not more than 4 dB below the prescribed

sound  pressure level on the axis at any direction in the horizontal plane within ±45 degrees of the axis.

The sound pressure level at any other direction in the horizontal plane shall be not more than 10 
dB below the  prescribed sound pressure level on the axis, so that the range in any direction will be at 
least half the range  on  the  forward  axis.  The  sound  pressure  level  shall  be  measured  in  that 
1/3rd-octave band which determines the audibility range.

(e)

Positioning of whistles

When a directional whistle is to be used as the only whistle on a vessel, it shall be installed with 
its maximum intensity directed straight ahead.

A whistle shall be placed as high as practicable on a vessel, in order to reduce interception of the 
emitted  sound by obstructions and also to minimize hearing damage risk to personnel. The sound

pressure level  of the vessel’s own signal at listening posts shall not exceed 110 dB (A) and so far as

practicable should not exceed 100 dB (A).

(f)

Fitting of more than one whistle

If whistles are fitted at a distance apart of more than 100 metres, it shall be so arranged that they are 
not sounded simultaneously.

(g)

Combined whistle systems

If due to the presence of obstructions the sound field of a single whistle or one of the whistles referred 
to in paragraph l(f) above is likely to have a zone of greatly reduced signal level, it is recommended 
that a  combined whistle system be fitted so as to overcome this reduction. For the purposes of 
the Rules a  combined whistle system is to be regarded as a single whistle. The whistles of a combined 
system shall be located at a distance apart of not more than 100 metres and arranged to be sounded 
simultaneously. The frequency of any one whistle shall differ from those of the others by at least 10 
Hz.

2.

Bell or gong

(a)

Intensity of signal

A bell or gong, or other device having similar sound characteristics shall produce a sound pressure 
level of not less than 110 dB at a distance of 1 metre from it.

(b)

Construction

Bells and gongs shall be made of corrosion-resistant material and designed to give a clear tone. 
The  diameter of the mouth of the bell shall be not less than 300 mm for vessels of 20 metres or more 
in length. Where practicable, a power-driven bell striker is recommended to ensure constant force 
but manual operation shall be possible. The mass of the striker shall be not less than 3 per cent of the 
mass of the bell.

3.

Approval

The construction of sound signal appliances, their performance and their installation on board the 
vessel shall be to the satisfaction of the appropriate authority of the State whose flag the vessel is 
entitled to fly.

ANNEX IV 
Distress signals

1.

The following signals, used or exhibited either together or separately, indicate distress and 
need of assistance:

(a) 
(b) 
(c) 
(d)

(e)

(f)

(g)

(h)

(i)

(j)

(k)

(l)

(m)

(n)

(o)

a gun or other explosive signal fired at intervals of about a minute;

a continuous sounding with any fog-signalling apparatus;

rockets or shells, throwing red stars fired one at a time at short intervals;

a signal made by radiotelegraphy or by any other signalling method consisting of

the group . . . _ _ _ . . . (SOS) in the Morse Code;

a signal sent by radiotelephony consisting of the spoken word “Mayday”;

the International Code Signal of distress indicated by N.C.;

a signal consisting of a square flag having above or below it a ball anything resembling 
a ball;

flames on the vessel (as from a burning tar barrel, oil barrel, etc.);

a rocket parachute flare or a hand flare showing a red light;

a smoke signal giving off orange-coloured smoke;

slowly and repeatedly raising and lowering arms outstretched to each side;

the radiotelegraph alarm signal;

the radiotelephone alarm signal;

signals transmitted by emergency position-indicating radio beacons;

approved    signals    transmitted    by    radio  communication    systems,    including 
survival  craft  radar transponders.

2.

The use or exhibition of any of the foregoing signals except for the purpose of indicating 
distress and need of assistance and the use of other signals which may be confused with any of 
the above signals is prohibited.

3.

Attention is drawn to the relevant sections of the International Code of Signals, the Merchant 
Ship Search and Rescue Manual and the following signals:

(a)        a  piece of orange-coloured  canvas  with  either a  black  square  and  circle  or  other

appropriate symbol (for identification from the air);

(b)        a dye marker.